Reprogramming a Metrologic Scanner

- You will need the MS951 Hand-Held Laser Scanner Programming Guide.

- When you program the scanner, if you delay scanning a programming code for more than about 5 seconds, the programming mode may be canceled and you’ll have to start over. Don’t fear – you can always return the scanner to factory default status – that’s the first action anyway. Just remember that programming involves ENTERing programming mode by scanning the ENTER/EXIT barcode, then scanning the specific program action barcode, and finishing the programming by scanning the ENTER/EXIT barcode again.

- Setting to factory default (pages 2-3)

- Scan ENTER/EXIT PROGRAM MODE on page 2

- Scan RECALL DEFAULTS on page 3

- Scan ENTER/EXIT PROGRAM MODE on page 2 again to finish.

- Same Symbol Re-Scan (pages 4-5)

- Scan ENTER/EXIT program mode on page 4

- Scan SHORT SAME SYMBOL RE-SCAN on page 5

- Scan ENTER/EXIT on page 4 again.

- Minimum Code Length for all code types (pages 32-33)

- Scan ENTER/EXIT on page 32

- Scan MINIMUM 1 CHARACTER on page 33

- Scan ENTER/EXIT on page 32 again.

- ScanPal Data Collector  (for the scanner attached to the ScanPal only - pages 48-49)

- Scan ENTER/EXIT on page 48

- Scan DEFAULT TO SCANPAL on page 49

- Scan ENTER/EXIT on page 48 again.