Easy Step By Step Touch Monitor Installation Instructions

Tools you may need: small knife or scissors, 1 small screwdriver, 1 medium

sized Philips head screwdriver, and possibly 1 medium sized regular (pan

head) screwdriver, and a couple pieces of tape.

1. Carefully remove your new touch monitor from the box. Save the box and

all the packing: you'll use it to ship your old monitor back to Club Data.

2. Carefully remove all plastic wrapping. Be careful not to cut any of the

cables coming out of the back of the monitor! Inspect the case and screen

-- make sure no damage occurred from shipping. Attach the swivel base -- it

snaps in place.

3. Make sure your computer is powered OFF, BUT LEAVE IT PLUGGED INTO THE

WALL! With a screwdriver, detach your old monitor from the computer. Make

note of the connectors into which the monitor was plugged. IMPORTANT: Mark

these with pieces of tape so you'll remember where it was plugged in.

Unplug the power cord from the back of the monitor. (The old power cord

will work with the new monitor so you don't have to replace it unless you

want to. A new power cord came with the new monitor.)

4. Remove the screws in the back of the computer case to remove the cover.

These screws are normally found in each corner of the back and possibly one

near the middle top of the computer's back side. Set these aside for now.

5. CAREFULLY, slide the cover off the computer. Sometimes they fit snugly,

and you may have to wiggle it a little to loosen it. Don't knock or hit it!

Remove the cover completely and set it aside.

6. Inside along the back side of the computer are several interface "cards"

which help make the computer work. Your old touch monitor was plugged into

two of them. You need to remove the mounting screws which hold these two

cards in (at the very back edge of the card against the back of the

computer). Save these screws. Then firmly (but carefully!) grasp one card

at a time and lift it out of its slot. They fit snugly, so you may have to

wiggle the card a bit from front to back (not sideways!) to loosen it. Put

each of the cards in one of the bubble wrap pouches we sent you and wrap it

with a rubber band. Set both aside for now -- you'll be shipping these back

to us with the old monitor.

7. Into one of the slots where a card was, insert the new VGA card. Make

sure it goes all the way in. Replace the mounting screw.

8. Into the slot from which you removed the other card, insert the spare

slot cover. It's important to cover all the slots so the computer gets

proper cross ventilation inside when the cover is on. Again, use the

mounting screw to secure the slot cover.

9. Carefully slide the computer case back on. Be careful not to catch the

case on any wires or cables -- sometimes there are a lot of them! Just tuck

them down carefully and slide the cover back on. Then replace the cover

screws again.

10. Plug your new touch monitor into the power cord.

11. Plug the monitor's VGA plug into the new VGA card you installed. The

VGA plug is the larger one which has about a dozen little gold pins inside

the plug. Be careful to plug it straight in, and then lightly twist the

screw mounts on each side of the plug.

12. Plug the other monitor cable (a longer, thinner one with a plug which

has 9 holes in the end) into one of the computer's back connectors which

accepts the plug. You can tighten the small plug screws to keep it firmly

attached for now, too.

13. Turn on the computer and monitor. You should see your startup menu

appear.

14. Get to the DOS prompt (if you use our menu system, type an asterisk to

drop to the DOS level). You should see something like C:> or D:> on the

screen.

15. Insert the Microcal diskette in drive A: and type A: to log the

diskette. The software on the disk is used to calibrate the touch monitor

so it will talk to your Touch-N-Sell software. You can do one of two things

with the disk: you can run the software directly from the diskette or you

can install the Microcal software on your hard disk. Type INSTALL for more

information on installing the software on your hard disk. FOR NOW, let's

run the software directly from the diskette.

16. With the A:> prompt on the screen, type MICROCAL to start the program.

Then press the SPACE bar to get the main menu screen.

17. Use the UP/DOWN arrow keys to highlight DIAGNOSTICS; press <ENTER>.

18. Highlight FIND TOUCH SCREEN; press <ENTER>.

19. Highlight SERIAL; press <ENTER>. If it finds the touch screen, it will

beep and display information on the right side of the screen in the STATUS

window. Write down what it says for PORT, BAUD and PARAMETERS.

20. If these settings are COM1, 4800 baud, and N72, that's good! Go to

step #21 below. OTHERWISE....

a) If it says COM2, it can probably work fine except if your receipt

printer is set to print to COM2, you'll have a conflict. The touch

monitor and receipt printer COM ports must be unique. Suggestion:

move the receipt printer to COM2 and plug the monitor into COM1.

Then in the TNS SETUP module, you'll have to tell the system to

print receipts to the proper COM port for the receipt printer.

b) If it says 9600, we need to run MICROCAL again to FORCE the

monitor to 4800 baud. Here's how:

(1) Start MICROCAL again from the floppy disk.

(2) Have it "find" the touch screen again as explained in steps

17-19 above.

(3) Highlight CALIBRATION and press <ENTER>. Touch the screen

where indicated.

(4) Highlight CONFIGURE and press <ENTER>.

(5) Highlight AUTO BAUD DETECTION ENABLE and press <ENTER>

(6) Highlight 4800 and press <ENTER>

(7) Then highlight selections to EXIT, all the way to exit the

program.

21. Now from the C:\ prompt, type CALIB to calibrate the screen for

Touch-N-Sell. It should work.

22. Lastly (sigh!) you need to check the batch files which start your

Touch-N-Sell system to make sure they set the COM ports to the proper

communications settings. Check the TNS.BAT file in the C:\ root directory.

Assuming that the touch monitor is plugged into COM1 and the receipt

printer is in COM2, here's what you'd want to find in the TNS.BAT file:

MODE COM1:4800,N,7,2

MODE COM2:9600,E,7,2

Make whatever edits you need to effect the changes.

23. Now comes the fine print. It's possible that by installing the touch

monitor, you may lose your report printer (temporarily) because some old

CGA monitor cards also included a parallel printer port. When you replace

such a CGA video card with a VGA card, you lose the printer port because

VGA cards don't have a printer port built into them. And depending on what

other interface cards you have installed, you may or may not need to add a

parallel printer card to get your parallel report printer working again.

Here's where you need to call us if you find extra cables that used to plug

into the back of your computer but which now don't have a home!

#