��������������������������������������������������������Ŀ

� This file is an exact copy of the online help facility �

� provided within microcal.                              �

����������������������������������������������������������

���������������������������������������������

���  ADJUST SENSITIVITY

���������������������������������������������

Adjusts the controller's ability to detect a

touch. For example, an application may require

a user to use latex gloves. Increasing the

controller's sensitivity will allow for

greater ability to detect the finger through

the latex glove.

NOTE: Increasing the sensitivity decreases the

touch screens accuracy.

���������������������������������������������

���  AUTOBAUD DETECTION

���������������������������������������������

You can disable or enable automatic baud rate

detection. Disabling the AutoBaud feature

allows the controller to automatically reset

itself when power is applied at the

predefined baud rate.

���������������������������������������������

���  BUS

���������������������������������������������

Search for a connected touch screen on

COM1 thru COM7.

NOTE: Searches COM1 thru COM7 as defined

by MicroTouch Bus Controllers

���������������������������������������������

���  CALIBRATION

���������������������������������������������

IMPORTANT: THIS OPTION WILL DELETE THE

CALIBRATION DATA CURRENTLY STORED IN THE

CONTROLLER.

CONTROLLER CALIBRATION

======================

Calibration is performed by the controller.

You must touch two points on the screen as

they appear (one in the bottom left, and one

in the top right).

The data from these touches is used by the

controller to establish the usable area

of the touch screen.

The calibrated area is stored in EEPROM on

the controller. Hence, this calibration is

non-volatile.

i.e. when the system is switched off,

the calibration is not lost.

���������������������������������������������

���  COLOR BARS

���������������������������������������������

Used for checking the color purity of the

display.

���������������������������������������������

���  COMMUNICATION PARAMETERS

���������������������������������������������

You can change the controller's default

communications parameters to parameters of

your choice.

Example: No Parity, 8 DataBits, 1 StopBit

���������������������������������������������

���  CONFIGURE

���������������������������������������������

This passes control to the menu of the

modifiable options.

The following options are available:

1) Adjust sensitivity. Adjusts the

controller's ability to detect a touch.

For example a person may be required to

use latex gloves. Increasing the

controller's sensitivity will allow for

a greater ability to detect the finger

through the latex glove.

2) Data protocol.  You can change the

default operating format and mode used on

power up.

Note: This is only required if the

default format and mode are not

used.

3) AutoBaud detection. You can disable or

enable automatic baud rate detection.

Disabling the AutoBaud feature allows

the controller to automatically reset

itself when power is applied at the

predefined baud rate.

4) Communication parameters.  You can change

the controller's default communications

parameters.

Example: No Parity, 8 DataBits, 1 StopBit

���������������������������������������������

���  DATA PROTOCOL

���������������������������������������������

Change the controller's default operating

format and mode used on power-up.

Note: This is only required if the default

format and mode are not used.

���������������������������������������������

���  DIAGNOSTICS

���������������������������������������������

This passes control to a menu of diagnostic

tools for the touch screen.

The following options are available:

1) Find touch screen.  This will search all

communication ports supported for a

connected touch screen.

2) Calibration.  This allows you to

calibrate the touch screen.

3) Noise check.  Checks the controller's

ability to ignore environmental noise,

and to also change the controller's

operating frequency to optimize its noise

rejection capability.

���������������������������������������������

���  DISABLE

���������������������������������������������

Configure the controller not to do

automatic baud rate detection.

���������������������������������������������

���  ENABLE

���������������������������������������������

Configure the controller to do automatic

baud rate detection.

���������������������������������������������

���  EXIT TO DOS

���������������������������������������������

Leave Microcal, and return control to DOS

���������������������������������������������

���  EXIT TO MAIN MENU FROM CONFIGURE

���������������������������������������������

Return to Microcals Main Menu.

���������������������������������������������

���  EXIT TO MAIN MENU FROM DIAGNOSTICS

���������������������������������������������

Return to Microcals Main Menu.

���������������������������������������������

���  F1 - HELP

���������������������������������������������

Pressing this key will give help information

about the menu option currently pointed at

(i.e.  context sensitive help).

For  example, suppose the Main Menu is

displayed, and you need help about the

'Re-Configure' option. Point the menu cursor

at this option, and then press F1.

���������������������������������������������

���  F2 - SELECT HELP LEVEL

���������������������������������������������

Allows you to specify the level of help you

require.

You may choose one of three levels of help:

Reminder - a short description of the

function. This level is suitable if you

simply  need a short reminder of what a

particular function is.

Summary - a more  detailed  description  of

the function.

Detailed description - This level is useful

if you are unfamiliar with Microcal. It

displays a full description of the function

in question.

���������������������������������������������

���  F3 - DRAW

���������������������������������������������

Allows you to check the touch screen by

'finger painting' on the screen.

The following keys are active during draw:

1 - 3 : change color (CGA);

9 : change color palette (CGA);

1 - 9 : change color (EGA/VGA);

g or G: draw 10x10 grid over drawing area

Space : clear screen;

ESC   : exit drawing mode.

���������������������������������������������

���  F4 - TERMINAL

���������������������������������������������

The Terminal utility allows touch screen

commands to be entered directly via the

keyboard.

When a character is entered, it is displayed

on the left half of the terminal screen. It

is then transmitted to the touch screen at

the displayed communications rate.

If the touch screen responds, it will be

displayed on the right half of the screen.

Touch coordinate responses are also

displayed on the right.

Characters with an ASCII value of less then

32 or greater than 127 will be displayed and

will appear as the shapes documented by IBM.

It may be useful to obtain a copy of the IBM

character set.

Carriage Return and Line Feed characters

are not displayed but will move the cursor.

Carriage return will also cause a line feed.

���������������������������������������������

���  F5 - VIDEOMODE

���������������������������������������������

Selects the current video mode from a list

of available video modes.

If the video mode is changed, the touch

screen must be recalibrated, as the display

areas for the various video modes are

different.

���������������������������������������������

���  F6 - RESETSTATUS

���������������������������������������������

Resets  Microcal  to its initial state (i.e.

all menu options reset, comms parameters set

to defaults, status box set to 'Search not

active'.

���������������������������������������������

���  FACTORY DEFAULT

���������������������������������������������

Configures the controller's data protocol

to use format decimal and mode stream.

���������������������������������������������

���  FIND TOUCH SCREEN

���������������������������������������������

This scans the communication ports for a

connected touch screen, operating with any

communication parameters.

If a touch screen is found, it is reset.

The status box is then updated with the

correct communication parameters, the

hardware and firmware version.

Note that this option does not change the

controller's active communication parameters:

it changes the computer's communication

parameters to match the controller.

���������������������������������������������

���  HELP LEVEL 1 : REMINDER

���������������������������������������������

Sets the help level to 1 (reminder).  When

you select help, a short reminder of the

subject chosen will be printed.

���������������������������������������������

���  HELP LEVEL 2 : SUMMARY

���������������������������������������������

Sets the help level to 2 (summary).  When

you select help, a detailed summary of the

subject chosen will be printed.

���������������������������������������������

���  HELP LEVEL 3 : DETAILED DESCRIPTION

���������������������������������������������

Sets the help level to 3 (detailed

description). When you select help,

a detailed explanation of the chosen

subject will be given.

���������������������������������������������

���  INTRODUCTION TO MICROCAL

���������������������������������������������

����������������������������������������������

Introduction

����������������������������������������������

Microcal is a program designed to allow you to

calibrate and test monitors fitted with

MicroTouch touch screens.

With Microcal you can:

Calibrate the touch screen controller to

your own requirements.

Perform more detailed tests on the accuracy

of operation of your touch screen.

Microcal is designed to be easy to use.

It will give you plenty of help,

suggest which step to take next, and will try

to locate problems for you.

As you perform the various tests in Microcal,

you will notice the menu bar moves by itself.

This is Microcal's way of suggesting which

step you might take next.

����������������������������������������������

Screen layout

����������������������������������������������

The  Microcal screen is split into several

distinct areas :

� Menu.   The current menu of options is shown

on the left of the display.

� Status.  The status box shows the current

RS-232 settings,  and the reset status of

the touch screen. The current screen mode,

video card detected and help level are

also shown.

� Function key line. This line is displayed

at the very bottom of the screen, and shows

the various function keys available.

When a new menu page is selected, it is shown

on top of the previous one, so you can

keep track of 'where you are'.

����������������������������������������������

Using Microcal

����������������������������������������������

To select a menu option, use the cursor up or

down keys to point to an option, then press

ENTER to select it.

When the function key line is shown, an 'F'

key may be selected instead of a menu option.

To exit from any option or menu, press ESC.

����������������������������������������������

Help

����������������������������������������������

A comprehensive on-line help facility is

included in Microcal. You can get help in two

ways :

1) Help index. If you press '?', an index of

all the help subjects is displayed. Use the

cursor keys to point at the one you want,

then press the <ENTER> key.

2) Context sensitive help. You can get help

about the current menu you are in by simply

pressing  F1 or the 'H' key.

Help information will be displayed about

the menu option currently pointed at.

The level of help given can also be defined

by pressing F2 ('Help level'). Three levels

of help are available :

1) Reminder.  A short reminder of the menu

function is displayed.

2) Summary.  A more detailed explanation of

the function is given.

3) Detailed  help. A full explanation of the

function is given, together with examples

where relevant.

���������������������������������������������

���  NOISE CHECK

���������������������������������������������

Determine how much electrical noise is

generated by the CRT in the selected

VideoMode that will be used by the application

and adjust the controller's operating

frequency to optimize noise rejection.

The VideoMode can be selected at the Main

Menu with the F5 function key.

���������������������������������������������

���  SELECT PROTOCOL

���������������������������������������������

Select the controller's data protocol from

a list of different formats and modes.

���������������������������������������������

���  SERIAL

���������������������������������������������

Search for a connected touch screen on

COM1 and COM2.

���������������������������������������������

���  STATUS BOX

���������������������������������������������

Shows touch screen status and current

communication parameters.

The status box is shown on the right of the

screen.

The box is split into three sections:

Touch screen status:  This box shows the

controller's output identity, hardware

version, firmware version and the results

of the self-test.

Comms parameters:  This shows the current

communication parameter settings i.e.

active port

baud rate

data bits

stop bits

parity

These parameters indicate the working

parameters of your touch screen.

Display mode:  This box shows the video card

detected, the current video mode and the

current help level.

Note that if the display  mode  is  changed,

the touch screen should be recalibrated,

as the display area is different in each

mode.